Flutter Project Setup Instruction (Ubuntu)

For

Plant Diagnosis System

# System Requirements

To install and run Flutter, your development environment must meet these minimum requirements:

- Operating Systems: Linux (64-bit)

- Disk Space: 600 MB (does not include disk space for IDE/tools).

- Tools: Flutter depends on these command-line tools being available in your environment.

  - bash

  - curl

  - file

  - git 2.x

  - mkdir

  - rm

  - unzip

  - which

  - xz-utils

  - zip

- Shared libraries: Flutter test command depends on this library being available in your environment.

  - libGLU.so.1 - provided by mesa packages such as libglu1-mesa on Ubuntu/Debian

Download Flutter SDK

    - Download the following installation bundle to get the latest stable release of the Flutter SDK:

https://flutter.dev/docs/get-started/install/linux

    - Extract the file in the desired location, for example:

Create new folder in home directory “development” and open terminal then type:

$ cd ~/development

To go to the development directory

Now use below command for extracting the downloaded SDK.

$ tar xf ~/Downloads/flutter_linux_1.17.3-stable.tar.xz

And go to development=>flutter=>  you find those files and folder

# Update Path:

- Determine the directory where you placed the Flutter SDK. You need this in Step 3.

- to edit $HOME/.bashrc. use this command: $ gedit ~/.bashrc

it will open the .bashrc file.

- Add the following line and change [PATH_TO_FLUTTER_GIT_DIRECTORY] to be the path where you cloned Flutter’s git repo at the end of the .bashrc file:

$ export PATH="$PATH:[PATH_TO_FLUTTER_GIT_DIRECTORY]/flutter/bin"

It should be like this

And save and close the file.

- Run source $HOME/.<rc file> to refresh the current window, or open a new terminal window to automatically source the file.

- Verify that the flutter/bin directory is now in your PATH by running:

$ echo $PATH

Verify that the flutter command is available by running:

$ which flutter

# Run Flutter Doctor

- Run the following command to see if there are any dependencies you need to install to complete the setup (for verbose output, add the -v flag):

$ flutter doctor

If you got “Error: unable to find git in your path ” this error you will need to  install git in your system using this command

$ sudo apt-get install git

- Now flutter doctor command will work and if Android studio is already installed it will show this

# Android Setup: Install Android Studio

- Download and install from:  https://developer.android.com/studio

- Extract the zip file and go to the bin directory and type:  ./studio.sh

- It will go through the ‘Android Studio Setup Wizard’.

- Select Do not import settings and click Ok.

- Procced to setup

- Select standard option

- It will download all required files on its own to complete the setup

- Click Finish to complete the Android Studio Setup

- This installs the latest Android SDK, Android SDK Command-line Tools, and Android SDK Build-Tools, which are required by Flutter when developing for Android.

- After open the Android studio this window will appear:

Warning: In Android Studio 3.6 or later, it needs to manually add the old version of the Android SDK Tools for Flutter to work. To do this:

- Open the Android Studio SDK Manager

- In the Android SDK tab, uncheck Hide Obsolete Packages

- Check Android SDK Tools (Obsolete)

The image below shows the appropriate settings:

This is a known issue that will be addressed in an upcoming version of Flutter.

# Install the Flutter and Dart plugins

To install these:

- Start Android Studio.

- Open plugin preferences (Configure > Plugins as of v3.6.3.0 or later).

- Search and select the Flutter plugin and click Install.

- Click Install when prompted to install the Dart plugin.

- Click Restart IDE

- Android Studio will start again and you can create or import project from here.

# Set up Android Device:

To run and test Flutter app on an Android device, It needs an Android device running Android 4.1 (API level 16) or higher.

- Enable Developer options:  go to “Settings  > About Phone”

- Tap  5-7 times continues on ‘Build Number’ (It will enable developer option for Project Run)

- After tapping, it will show a toast ‘You are in developer mode’.
If you did this already, you will get the toast ‘No need, you are already a developer’ .

- Again, Go to “Settings > Developer Option”

- Enable USB debugging on your device. Tap on the ‘USB debugging’ to enable it.

- And If the alert dialogue is opened then simply click on the marked ‘OK’ button

- Using an USB cable, plug the phone into the computer. If prompted on the device, authorize the computer to access that device by click on the marked ‘OK’ button.

Windows-only, Install the Google USB Driver if it fails to detect automatically. Help guide is available in- https://developer.android.com/studio/run/win-usb

- In the terminal, run the flutter devices command to verify that Flutter recognizes the connected Android device.

# Set up the Android Emulator:

To prepare to run and test Flutter app on the Android emulator, follow these steps:

- Enable VM acceleration on your machine.

- Start Android Emulator.

- Click on the marked Icon as below image

- Click on the Create Virtual Device

Or, Launch Android Studio > Tools > Android > AVD Manager and select “Create Virtual Device”. (The Android submenu is only present when inside an Android project.)

- Choose a device definition and select Next.

- Select one or more system images for the Android versions, and select Next. An x86 or x86_64 image is recommended.

- Under Emulated Performance, select Hardware - GLES 2.0. Verify the AVD configuration is correct, and select Finish.

- In Android Virtual Device Manager, click Run from the toolbar.

- The emulator starts up and displays the default canvas for selected OS version and device.

# Build and Run the Project in the Emulator:

- Clone the git repository from https://github.com/tsiAILAB/Plant-Pathology.

Please download the zip and extract.

- Open the project in android studio marked menu.

- Select the project folder which is downloaded from Git Repository. The project should be this location of coned GIT  “\plant-pathology\flutter\plantpathologyandroid”

- Download all dependencies by clicking Get dependencies

- Click (<no devices>)-highlighted in below image, and select device/emulator.

- Click on the Run icon to run the project in the device/emulator (shown in the image given below).

# Build APK

- To build apk go to build menu of Android Studio

- Select “Build Bundle(s)/ APK(s)”

- Build APK(s)

- An APK will generated in the build folder

# Run the own Built App

  - Copy the app-debug.apk file to android device.

  - Select the file to install.

| Installation Tap to INSTALL to get the app installed in your device. | Installation After getting the Installation confirmation, tap on OPEN button to start the app |
| --- | --- |